Introduction

============

idetool binary should be located in your SYS:C/ directory. You should

call idetool from a shell, using command line arguments as per the

reference below.

idetool is a command line tool which allows querying and modifying

internal parameters for the AmigaOS4 ATA/ATAPI device drivers :

- a1ide.device, which controls the VIA686B PATA IDE controller,

- sii0680ide.device, which controls Silicon Image SiI0680(A)-based

PCI PATA IDE controller,

- sii3112ide.device, which controls Silicon Image SiI3112(A)-based

PCI SATA IDE controller.

- sii3114ide.device, which controls Silicon Image SiI3114-based

PCI SATA IDE controller.

- sii3512ide.device, which controls Silicon Image SiI3512-based

PCI SATA IDE controller.

- lsi53c8xx.device, which controls LSI Logic 53c8xx-based SCSI

PCI controllers.

idetool can also be used to examine / modify any PCI device

configuration and I/O space (only use this feature if you really

know what you are doing). A word on PCI device configuration space :

- All PCI devices are uniquely identified by a Vendor and a Product

code. Those are 16 bit figures, which can be represented as 4 hex

digit values. Example : the VIA IDE controller is identified by

vendor code 0x1106 and product code 0x0571.

- All PCI devices expose on the bus, 256 bytes of configuration data

called the "configuration space". This configuration space contains

a part which is rigidly defined in the PCI specification, and a part

which structure is up to the device manufacturer. In the standardized

part, we find things like the vendor & product codes, the PCI latency

setting for the device, the interrupt pin / line settings for the

device etc.

- Additionnally to this configuration space, each PCI device can have

up to 5 address ranges, which can either be "IO space" or "memory

space", for whatever usage. Example : a graphics card will probably

have a memory mapped area which contains a linearly addressed image

of its framebuffer memory.

The purpose of the PCI configuration and IO manipulation features in

idetool is primarily to be able to finetune the IDE device operation,

but it can also be used as a general purpose diagnostic / tuning tool

for all other PCI devices (USB, Ethernet, Audio, Graphic cards etc.).

Command reference

=================

idetool -<cmd1> [cmd arg1] ... [cmd arg n] -<cmd2> [cmd arg1] ... [cmd arg n] ...

Where <cmd> is a command, and [cmd arg1] ... [cmd arg n] are the command args

General commands :

-v (no args) : tell version

-c (no args) : tell CPU configuration

OS4 device unit commands :

-l devname : tell unit info for all device units

-u devname unit : tell unit info for unit 'unit'

-d devname unit : tell drive info for unit 'unit'

-r devname unit : tell raw drive info for unit 'unit'

-x devname unit mode : put drive 'unit' into transfer mode 'mode'

-p devname unit mode : put drive 'unit' into power mode 'mode' (2: spin up, 3: spin down, 5: sleep forever)

-i devname unit mode : put drive 'unit' into interrupt mode 'mode' (0 : don't use irq's, 1 : use irq's)

-o devname unit : just open & close devname/unit

CD/DVD-specific device unit commands :

-j devname unit 'e'/'l' : eject/load disk in drive 'unit'

-m devname unit read_speed write_speed : set 'unit' medium read / write speed (1 .. n for x1 ... xn, or 0xFFFF for max.)

PCI chip config dump commands :

-w (no args) : tell VIA686b PCI IDE configuration

-s (no args) : tell SiI0680 PCI IDE configuration

-a (no args) : tell SiI3112 PCI IDE configuration

-b (no args) : tell SiI3114 PCI IDE configuration

-e (no args) : tell SiI3512 PCI IDE configuration

-f (no args) : tell LSI53cxxx PCI SCSI configuration

-t vendor device : tell PCI device 'vendor/device' configuration

PCI chip config patch commands :

-0 reg val : patch VIA function 0 (ISA) configspace byte 'reg' with value 'val'

-1 reg val : patch VIA function 1 (IDE) configspace byte 'reg' with value 'val'

-2 reg val : patch SiI0680 configspace byte 'reg' with value 'val'

-3 reg val : patch SiI3112 configspace byte 'reg' with value 'val'

-4 reg val : patch SiI3114 configspace byte 'reg' with value 'val'

-5 reg val : patch SiI3512 configspace byte 'reg' with value 'val'

-5 reg val : patch SiI3512 configspace byte 'reg' with value 'val'

-g reg val : patch LSI53cxxx configspace byte 'reg' with value 'val'

-8 vendor device reg : tell PCI device 'vendor/device' configspace byte 'reg' value

-9 vendor device reg val : patch PCI device 'vendor/device' configspace byte 'reg' with value 'val'

PCI chip I/O patch commands :

-6 vendor device reg : tell PCI device 'vendor/device' I/O byte 'reg' value

-7 vendor device reg val : patch PCI device 'vendor/device' I/O byte 'reg' with value 'val'

Beware : do not use this tool except if you really know what you are doing.

Note on inputting numeric values : if the first character is 0 and the second character is not 'x' or 'X', the string is interpreted as an octal integer; otherwise, it is interpreted as a decimal number. If the first character is '0' and the second character is 'x' or 'X', the string is interpreted as a hexadecimal integer.