%N sigest

%D Estimate the rms level in an image.

%P rjs

%: map analysis

%B

SIGEST is a Miriad task which attempts to estimate the rms noise

level in an image. It does this by determining statistics of a

clipped version of the image data. Based on the statistics of

clipped Gaussian noise, the rms is then estimated. This

assumes that the image pixels, after clipping, are dominated by noise.

SIGEST will work poorly on images containing significant sidelobes.

%A in

The input image dataset. No default.

%A clip

The clip level to use. Pixels are excluded from the calculation

if their absolute value is greater than the clip value. Ideally

the clip should be about 2 to 3 times the rms noise level.

The default uses an initial estimate of the rms to set the clip

at 2.5 times the rms.

%A region

The standard image region of interest of the input. See the help

on "region" for more information. The default is the entire image.