%N pntgen

%D Generate fake pointing data.

%P mchw

%: pointing analysis

%B

PNTGEN generates fake pointing data and calculates the errors

which arise from using linear pointing equations.

The default is to print out the pointing errors and the

difference in arcmin from the linear pointing equations.

If an output file is specified then PNTGEN also generates

a file containing npts of pointing data at random

azimuth and elevation points within azrange and elrange.

%A collim

Collimation angle in arcmin between telescope axis and

normal to elevation axis. Default=0.

%A tilt

Tilt angle in arcmin of azimuth axis. Default=0.

%A npts

Number of samples of pointing data generated. Default=100.

%A azrange

Azimuth range of pointing data generated. Minimum and

maximum values in degrees. Default=-85,85.

%A elrange

Elevation range of pointing data generated. Minimum and

maximum values in degrees. Default=5,85.

%A out

Output filename for pointing data.

e data in CARMA 12sep05 ddmmmyy before the UT format