%N obsrms

%D Calculate theoretical rms for telescope array observations.

%P mchw

%: image analysis

%B

OBSRMS - Calculate theoretical rms for telescope array observations.

The MIRIAD task TELEPAR provides typical values for Tsys and Jy/K.

for several observatories.

%A tsys

SSB system temperature (Kelvin). Default=300 K.

Tsys includes the loss through the atmosphere.

%A jyperk

Antenna gain in units Jy/K. Default=150 Jy/K.

%A antdiam

Antenna diameter (m), and aperture efficiency can be used to calculate

jyperk. If only one value is given then the aperture efficiency used

is 0.6. Default=0,0.6 i.e. use jyperk value.

%A lambda

wavelength in millimeters. Default=3 mm.

%A freq

freq (GHz) can be used to compute lambda. Default=0 GHz, uses lambda.

%A theta

Beamsize in arcsec. Two values may be given for the beam FWHM.

If only one value is given it is used for both axes. Default=1,1

%A nants

Number of antennas in array. Single antenna is OK too. Default=9.

%A bw

Bandwidth in MHz used for continuum observations. Default=800 MHz.

%A deltav

Velocity resolution in km/s  can be used to compute bandwidth for

spectral line observations. Default=0 km/s, i.e. use bandwidth value.

%A inttime

Total Integration time on source in minutes. Default=1 minute.

%A coreta

Correlator efficiency factor. A perfect correlator has coreta=1.

2-, 3-, and 4-level correlators have coreta=0.64, 0.81, and 0.88

respectively. (see Thompson, Moran, and Swenson, p229).

Default=0.88

%A rmsphase

RMS phase noise (degrees). Phase noise reduces the signal, effectively

increasing the observed RMS noise in the data by a factor

exp(rmsphase(radians)**2/2.). For strong sources atmospheric

phase noise can be removed by selfcalibration.

Default=0. degrees. i.e. perfect data.