%N atxy

%D Apply on-line XY phase measurements to Miriad data.

%P rjs

%: uv analysis

%B

ATXY corrects data for XY phase errors. It determines the XY phase

measurements, either from the xyphase visibility variable or from

the text file written by AIPS ATLOD, averages them, and then

applies them to the data. The averaging step consists of finding

the median XY phase in a given interval.

%A vis

The names of the input uv data sets. No default.

%A xyphase

If the data were loaded with Miriad ATLOD, then they should contain

the on-line XY phase measurements in the xyphase variable. Howver

if the data were loaded with AIPS (and then transferred to Miriad

via FITS files), then you must give the name of the XY phase

text file written by AIPS. Generally it will be XYPHS_xx, where

xx is an AIPS user number.

%A refant

Antennas to apply XY phase corrections to. Generally it is best to

apply corrections to only a single antenna -- the antenna which you

have the greatest confidence in the XY phase measurements. Often

this antenna will be the reference antenna during the observation.

If you apply the XY phase to a subset of antennas, one of these

antennas should be used as the ``refant'' antenna to GPCAL.

No default.

%A interval

This gives the length of an averaging interval, in minutes. The

default is 30 minutes, but larger values may be useful.

%A break

The times, in the normal Miriad format, where there are

break points. A ``break point'' gives a time where some form of

instrumental glitch took place, and the xyphases jumped. By

giving a break point, you prevent a solution interval from spanning

this time.

%A sideband

Sideband indicators. If the input was loaded with AIPS ATLOD, and

if you averaged it into a channel-0 data-set, you must give the

sideband indicator (+1 or -1) for each IF. This information is needed

to convert between the AIPS and Miriad XY phase convention.

If the input is still multi-channel data, or you used Miriad ATLOD,

it is wiser to let this parameter default. Note that ATXY cannot

cope with channel-0 data-sets when the sideband indicators

change with time.

%A out

The name of the output uv data set. No default.

%A options

Extra processing options. Several options can be given,

separated by commas. Minimum match is supported. Possible values

are:

srcbreak  Generate break points every time there is a change

in source.

undo      Apply the negative of the XY phase correction. There

should be no sensible reason to ever use this option.

reweight  Reweight the data (in the lag domain) to minimise the

so-called Gibbs phenomena and to help localise

interference.