%N dwell

%D Calculate dwell time for given array configuration

%P mwp

%: utility

%B

DWELL - Calculate the dwell time for a given array configuration.

The dwell time is the maximum amount of time that can be

spent on a given field before the longest baseline

rotates to a completely independent visibility point. It is

given by

tdwell = (43200/pi)*(D/L)*(1/N)

where D is the antenna diameter, L is the longest baseline, and

N is the number of fields.

This task can be used to help maximize the on-source time and

increase observing efficiency. It is of most use when planning

mosaic observations.

%A ary

The name of a text file containing the antenna positions

(in nanosec and either topocentric or geocentric coordinates).

Can be either same format as for uvtrack ("ary format") or uvgen

("ant" format). No default.

%A format

Format for array file, "ant" or "ary", indicating styles

of $MIRCAT/*.ant[pos] or  $MIRCAT/*.ary, respectively.

Default is "ant".

%A nfields

Number of fields in the mosaic. No default.

%A antdiam

Antenna diameter (m). Default=6.1

%A options

verbose  Print out the antennna pairs and baseline lengths