%N linecal

%D Estimate antenna gains from line length measurements.

%P mchw

%: calibration

%B

LINECAL is a Miriad task to estimate the instrumental phase

drifts due to line length changes. The measured phases are

written as antenna gains and can be manipulated and displayed

using the GP routines and BEE. The resulting gains can be plotted

with GPPLT and smoothed to reduce the noise with GPAVER.

The gains for bad line length phases can be edited with GPEDIT,

or with BEE if you want to do fancy things.

The line length gains are applied when copying or plotting

the data.

NOTE: the linelength data are stored as an antenna based

UV variable called phasem1, in radians. But note that this

variable is not removed by any subsquent uvcat, and applying

linecal twice is probably not a good thing.

%A vis

Name of input visibility data file. No default.

The visibility data must be in time order.

%A out

Optional name of the output calibration file.

The default is to write the gain corrections into the input

visibility file.

%A freq

Optional Frequency (in GHz) to use to convert a cable (in ms)

measurement (in ns) to phasem1 (in radians) for linelength.

Normally this option is not used, since phasem1 was derived

by using the LO1 frequency. This option has been added for

debugging the cable to phasem1 conversion.

Usually this option does not make sense for CARMA as LO1 is

doppler  tracked and this freq= would not be. For SZA this is

appropriate, as they do not doppler track.

For SZA data freq=35.938 should be used.

Default: not used, and phasem1 is used to build the gain table.