%N xyphase

%D Determine XY phase corrections from on-line measurements.

%P rjs

%: uv analysis

%B

XYPHASE is a MIRIAD task, which determines XY phase corrections

from the on-line XY phase measurements. This can calculate

an antenna gain and/or a bandpass function.

This cannot cope with multi-IF data-sets. Bad solutions will result

if you attempt to use it for this data.

The input data should contain either the "xyphase" uv variable

(the on-line XY phase measurement), and/or autocorrelation data. The

autocorrelation data is required if the bandpass is being solved for.

Note that the "xyphase" variable is lost if the data enters Miriad

via FITS (unless ATMERGE is used). The autocorrelation data is

currently not written when using the standard correlator

configurations.

%A vis

The input visibility data-set. No default. This should contain

raw ATCA polarisation data (i.e. XX,YY,XY,YX correlations).

%A select

Normal uv selection criteria. The default is to select all data.

Polarisation selection should not be used.

%A refant

The reference antenna. By default XYPHASE computes the gains and

bandpasses for all antennae. By specifying a reference antenna,

XYPHASE finds solutions for only the reference antenna, and sets the

other phases to zero. This is as other programs (such as GPCAL and

MFCAL) can determine all but the reference antennas XY phases.

%A interval

This gives one or two numbers, both given in minutes, both being used

to determine the interval over which the antenna XY phase is assumed

to remain constant. The first number gives the maximum length of

a solution interval. The second number gives the maximum gap size in

a solution interval. A new solution is started when either the

maximum time length is exceeded, or a gap larger than the max gap is

encountered. The default is a maximum length of 5 minutes, and a

gap size which is the same as the maximum length.

%A break

The times, in the normal Miriad format, where to insert the

break point. A ``break point'' gives a time where some form of

instrumental glitch took place, and the xyphases jumped. By

giving a break point, you prevent a solution interval from spanning

this time, and you inhibit gain interpolation across the time.

%A options

Extra processing options. Several options can be given, separated

by commas. Minimum match is used.

noxy    Do not determine the antenna XY phases. By default

XYPHASE will solve for this when the "xyphase" variable

is present in the data.

nopass  Do not determine the XY bandpass function. By default

XYPHASE will solve for this when autocorrelations are

present in the data.