%N calred

%D Analyse flux densities of sources with poor phase stability

%P rjs

%: uv analysis, plotting

%B

CALRED is a program used to analyse flux densities of sources

where phase stability is poor. It uses either the amplitude of

the data or an averaged triple product.

%A vis

The input visibility datasets. Several datasets can be given.

%A select

Standard visibility selection. See help on "select" for more

information.

%A line

Standard visibility linetype. See the help "line" for more

information.

%A stokes

Normal Stokes/polaization selection. The default is to process

all parallel-hand polarisation.

%A interval

Seperate estimates of the various parameters are determined for

data over an interval. This parameter gives the interval, in

minutes. The default is 5 minutes.

%A options

triple    Do triple processing.

nocal     Do not perform gain calibration.

nopol     Do not perform polarisation calibration on the data.

nopass    Do not perform bandpass calibration on the data.

norm      Divide the noise estimates by the square root of the

number of points.