%N tmcal

%D Gain/phase/polarization calibration for SMA data.

%P rjs

%: calibration

%B

tmcal is a MIRIAD task which determines calibration corrections

(both antenna gains and instrumental polarisation characteristics).

%A vis

Input visibility data file. No default. The visibility data

must be in time order.

%A select

Standard uv selection. Default is all data.

%A line

Standard line-type specification. Multiple channels can be given.

Generally it is better to give multiple channels, rather than

averaging them into a `channel-0'. The default is all the channel

data (or all the wide data, if there is no channel data).

%A flux

The values of the I,Q,U Stokes parameters. If no values are

given, and it is a source known to tmcal, tmcal uses its known

flux as the default. If tmcal does not know the source, the

flux is determined by assuming that the rms gain amplitude is 1.

For a planet, the values are interpreted as brightness temperature

in Kelvin (planets are assumed to be unpolarised). Otherwise the values

are assumed to be in Janskys.

If the option `qusolve' is used, the given fluxes for Q and U are

used as the initial estimates.

%A refant

The reference antenna. Default is 1. The reference antenna needs to be

present throughout the observation, and so needs to be chosen with some

care. Any solution intervals where the reference antenna is missing are

discarded.

%A minants

The minimum number of antenna that must be present before a

solution is attempted. Default is 2.

%A interval

This gives the solution interval, in minutes. Antenna gains are assumed

to be constant over the solution interval.

%A tol

Error tolerance. The default is 0.001, which should be adequate.

%A options

These options determine what TMCAL solves for. Several options

can be given,separated by commas.

nopass     Do not apply any passband calibration.

qusolve    Solve for Q and U fluxes. Good parallactic

angle coverage is required for this.

twoleak    Solve for a "two part" leakage solution,

onegain    Solve for a single antenna gain, which is assumed to be

independent of polarisation. When time-multiplexing of a single

feed is used to measure two states (e.g. R and L), then it should

be accurate to assume the antenna gains to R and L are the same.