%N gpscal

%D Self-calibration for arrays with linear feeds

%P rjs

%: calibration, map making

%B

GPSCAL is a MIRIAD task to perform self-calibration of visibility data,

where the visibility data is produced by linear feeds, and the source

is moderately or strongly polarised. Either phase only or amplitude

and phase calibration can be performed. The inputs to GPSCAL include a

visibility data file, and model images. The models should be of

differing polarisations of the same pointing and frequencies.

GPSCAL then calculates the visibilities corresponding to the model,

accumulates the statistics needed to determine the antennae solutions,

and then calculates the self-cal solutions.

Note that while GPSCAL can be used for self-calibrating with a point

source model, task GPCAL is more flexible (and efficient) at doing

this. For a point source model, GPSCAL is preferable only if the

data-set is not in time order (GPCAL insists that it is) or the point

source is not at the observing center.

%A vis

Name of input visibility data file. No default.

%A select

Standard uv data selection criteria. The default is all data.

%A model

Name of the input models. Several models can be given, which differ

in polarisation type. Polarisation types can be I,Q,U or V. Any

polarisation types that are not given are assumed to be zero.

The different models should correspond to the same pointing

and range of channels. Generally the model should be derived (by

mapping and deconvolution) from the input visibility file, so that

the channels in the model correspond to channels in the visibility

file. Though the maps can be made using any linetype, generally

"channel" linetype will give best results (??). The units of the

model MUST be JY/PIXEL, rather than JY/BEAM. It should be weighted

by the primary beam. The task DEMOS can be used to extract primary

beam weighted models from a mosaiced image. If no models are given,

a point source model is assumed (see FLUX keyword).

%A clip

Clip level. For models of intensity, any pixels below the clip level

are set to zero. For models of Stokes Q,U,V, or MFS I*alpha models,

any pixels whose absolute value is below the clip level are set

to zero. Default is 0.

%A flux

If not model is given, then a point source model is assumed. This

keyword gives the flux of the point source model. Four values can be

given, corresponding to I,Q,U and V respectively. NOTE: The flux

of the model is not adjusted to match the flux of the data, so if

amplitude selfcalibration is being performed, the fluxes given should

accurately portray the fluxes of the source. The default is 1,0,0,0.

%A offset

This gives the offset in arcseconds of a point source model (the

offset is positive to the north and to the east). This parameter is

used if the MODEL parameter is blank. The default is 0,0. The

amplitude of the point source is chosen so that flux in the model

is the same as the visibility flux.

%A interval

The length of time, in minutes, of a gain solution. Default is 5,

but use a larger value in cases of poor signal to noise, or

if the atmosphere and instrument is fairly stable.

%A options

This gives several processing options. Possible values are:

amplitude  Perform amplitude and phase self-cal.

phase      Perform phase only self-cal.

noxy       Do not attempt to solve for the XY phase difference.

Normally GPSCAL attempts to solve for the XY phase

on all antennae except for the reference antenna.

The XY phase difference is assumed to be constant

throughout the observation.

xyvary     Allow the XY phase difference to vary from integration

to integration. If not specified, the XY phase is

constrained to be constant throughout the data-set.

xyref      Solve for the XY phase of the reference antenna. To

do this, the source should be strongly polarized (at

least 5%) and Q or U models should be provided. This

option cannot be used with ``noxy''.

mfs        This is used if there is a single plane in the input

model, which is assumed to represent the image at all

frequencies. This should also be used if the model has

been derived from MFCLEAN. You should specify the

LINE keyword if you use the mfs option.

noscale    Do not scale the gains. By default the gains are scaled

so that the rms gain amplitude is 1. Generally this

should be used if the model is believed to have the

correct flux density scale.

Note that "amplitude" and "phase" are mutually exclusive.

The default is options=phase.

%A minants

Data at a given solution interval is deleted  if there are fewer than

MinAnts antennae operative during the solution interval. The default

is 3 for options=phase and 4 for options=amplitude.

%A refant

This sets the reference antenna, which is given a phase angle of zero.

The default is antenna 3.

%A line

The visibility linetype to use, in the standard form, viz:

type,nchan,start,width,step

Generally if there is an input model, this parameter defaults to the

linetype parameters used to construct the map. If you wish to override

this, or if the info is not in the header, or if you are using

a point source model, this parameter can be useful.